Model 2050 Continuously Hinged / Electric

Introduction:

The following three (3) part specification offers the Standard and Optional features for the Model 2050 Continuously Hinged / Electric Operable Wall System.  The yellow highlighted areas in the specification indicate an Optional selection that is available based on your project requirements.

In order to assist you with the design criteria KWIK-WALL has provided a Product Guide and Track & Carrier Selection Chart for the Model 2050.

The Product Guide indicates the acoustical ratings (STC) available, and establishes the maximum partition height and width.

| MODEL 2050 PRODUCT GUIDE Standard Acoustical Substrate Construction |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | PANEL | MAXIMUM | MAXIMUM | MAXIMUM |
| STC | THICKNESS | PANEL WEIGHT | PANEL | WALL |
| RATING | (nominal) | lb. / ft.2 | HEIGHT | WIDTH |
| 42 | 3" [76] | 6.5 (32 kg/m2) | 14'-2" (4.32 m) | 42' 3 ¾” [12.9] |
| 45 | 3" [76] | 7.5 (36.6 kg/m2) | 14'-2" (4.32 m) | 42' 3 ¾” [12.9] |
| 49 | 3" [76] | 9.0 (44 kg/m2) | 14'-2" (4.32 m) | 42' 3 ¾” [12.9] |
| 50 | 3" [76] | 9.0 (44 kg/m2) | 14'-2" (4.32 m) | 42' 3 ¾” [12.9] |
| Note: Optional Wood Veneer or High Pressure Laminate only available as Acoustical Substrate Construction. |  |  |  |  |

| MODEL 2050 PRODUCT GUIDE Optional Steel Skin Construction |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | PANEL | MAXIMUM | MAXIMUM | MAXIMUM |
| STC | THICKNESS | PANEL WEIGHT | PANEL | WALL |
| RATING | (nominal) | lb. / ft.2 | HEIGHT | WIDTH |
| 49 | 3" [76] | 8.0 (39 kg/m2) | 14'-2" (4.32 m) | 42' 3 ¾” [12.9] |
| 51 | 3" [76] | 11.0 (54 kg/m2) | 14'-2" (4.32 m) | 42' 3 ¾” [12.9] |

* Estimated panel weights are for intermediate panels. Weight may vary due to substrate, size, or function of panel. Add 105 lbs [47kg] for pass door. Add 6 lbs [3kg] per lin. ft. height for expanders. Add 3.5 to 8 lbs [1.6 to 3.6kg] per lin. Ft. for track.

**Standard features can be modified, contact your Kwik-Wall distributor for the features you want.

***600 sq.ft. [55.74] maximum size

The Track & Carrier Selection Chart determines the appropriate track and carrier system based on the STC Rating and Panel Fabrication Height.

## Model 2050 Continuously Hinged / Electric Product Specification

## PART 1 – GENERAL SPECIFICATIONS

- 1.01 WORK INCLUDED

- A.	Operable Wall System shall be furnished, installed and serviced by wall manufacturer's authorized distributor, in compliance with the architectural drawings and specifications contained herein.

- 1.02 RELATED WORK

A.	Structural Support:  Structural support system required for suspending the operable wall shall be designed, installed and pre-punched by others, in accordance with ASTM E 557 and manufacturer's shop drawings.

- B.	Insulation:  Sound insulation and baffles for the plenum area above the track system, under the permanent floor, inside air ducts passing over or around the operable wall, and in permanent walls adjoining the operable wall system shall be by others, in accordance with ASTM E 557.

- C.	Opening Preparation:  Proper and complete preparation of the operable wall system opening shall be by others in accordance with ASTM E 557, and shall include floor leveling; plumbness of adjoining permanent walls; substrate and / or ceiling tile enclosures for the track system; and the painting and finishing of trim and other materials adjoining the head and jamb areas of the operable wall.

- 1.03 SYSTEM DESCRIPTION

A.	The operable wall system shall consist of Continuously Hinged panels that are electrically operated, featuring panels hinged together in a continuous panel train.

- B.	The operable wall system shall consist of acoustically rated panels tested in accordance with ASTM E 90 and ASTM E 413 test procedures, and shall have achieved a STC rating as specified herein (see "Acoustical Performance" article listed under Part 2 – Products).

- 1.04 QUALITY ASSURANCE

A.	The operable wall shall have been tested in an independent acoustical testing laboratory in accordance with ASTM E 90 and ASTM E 413 test procedures.

- B.	The operable wall panel construction and finish materials shall consist of Class A rated materials in accordance with ASTM E 84.

- C.	The operable wall shall be installed by the manufacturer's authorized distributor in accordance with ASTM E 557.

- 1.05 REFERENCES

A.	ASTM E 90:  Laboratory Measurement of Airborne-Sound Transmission Loss of Building Partitions.

- B.	ASTM E 413:  Determination of Sound Transmission Class (STC).

- C.	ASTM E 557:  Architectural Application and Installation of Operable Partitions.

- D.	ASTM E 84:  Surface Burning Characteristics of Building Materials.

- E.	ASTM A 653:  Specification for General Requirements for Steel Sheet, Alloy-Coated (Galvannealed) by the Hot Dip Process.

- F.	ASTM C 423:  Standard Test Method for Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method.

- G.	CCC-W-408A:  Federal Specification which applies to Vinyl Coated Wall Coverings.

- H.	CFFA-W-101-D:  Chemical Fabrics and Film Association Quality Standard for Vinyl Coated Fabric Wall Coverings.

- I.	NFPA 70:  Standard for the safe installation of electrical wiring and equipment.

- 1.06 SUBMITTALS

A.	Manufacturer shall provide written technical information and related detail drawings, which demonstrate that the products comply with contract documents for each type of operable partition specified.

- B.	Manufacturer shall provide detailed engineering drawings featuring track plan, panel elevation, horizontal and vertical details, wiring diagram and beam punching template as required.

- C.	Manufacturer shall provide written test report of the independent acoustical testing laboratory certifying the attainment of the specified STC rating, upon request.

- D.	Manufacturer shall provide written instructions specifying the proper operation and maintenance of the operable wall system.

- E.	Manufacturer shall provide a color selector demonstrating the manufacturer's selections of the specified finish material.  Samples shall consist of actual swatches of the specified finish material.

- 1.07 DELIVERY, STORAGE AND HANDLING

A.	Panels shall be individually wrapped in a protective plastic covering to keep panels clean during delivery, storage and handling.

B.	Panels shall be stored on edge and above the floor on cushioned blocking in a dry and ventilated area, protected from humidity and temperature extremes.

- 1.08 SEQUENCING / SCHEDULING

A.	Beam Punching:  Manufacturer shall provide beam punching template drawing detailing the anchor locations for the suspended track system (as required for Drop Rod Mounting), as required for the fabrication and installation of structural overhead support by others.

- B.	Track Installation:  Scheduling of operable wall track installation shall occur after structural overhead support has been properly and completely fabricated and installed by others.

- C.	Panel Installation:  Operable wall panel installation shall occur after fixed wall substrate construction is properly and completely installed by others, as required to protect panels from ongoing adjacent construction.

- 1.09 WARRANTY

- Manufacturer shall warrant each 2050 partition and its component parts to be free from defects in material and workmanship for a period of five (5) years from the date of delivery to the original purchaser, when installed by an authorized KWIK-WALL distributor.  (Contact your local KWIK-WALL distributor or KWIK-WALL Company for complete warranty information.)

## PART 2 – PRODUCT SPECIFICATIONS

2.01 ACCEPTABLE MANUFACTURER

- Operable walls shall be Series 2000, Model 2050 Continuously Hinged / Electric as manufactured by KWIK-WALL Company.

2.02 PANEL CONSTRUCTION

- Panel Dimensions:  Standard panel dimension shall be a nominal 3" [76.2] thick.

- Panel Frame:  Steel frame shall be 18-gauge galvanneal steel, which meets or exceeds ASTM A 653 requirements.  Frame shall be all-welded construction with steel corner supports and cross-bracing reinforcement.  Panel frame shall be Class A rated, fire retardant, non-combustible and non-corrosive in accordance with ASTM E 84.

- Panel Skins:  Panel skins shall be Class A rated in accordance with ASTM E 84.  Panel skin material shall consist of (select):

1.	Standard Steel Skins: consisting of minimum 22-gauge tension-leveled galvanneal steel, pressure laminated to a structural acoustical backer and mechanically-joined to the steel frame to form a rigid, unitized and structural panel.

2.	Optional Acoustical Substrate: consisting of structural acoustical substrate pressure laminated and     mechanically-joined to both sides of the steel frame to form a rigid, unitized and structural panel.

- Panel Hinges:  Panel hinges shall be architectural grade, full leaf butt hinges.  Hinges shall be attached to steel 		frame utilizing a steel mounting bracket welded to frame.

- Panel Weight:  Maximum panel weight shall be 6.5 – 11 lb. / ft.2 (32 – 54 kg / m2) depending on STC rating, size and options selected.

2.03 OPERATION

- Operation:  Operation shall be Continuously Hinged / Electric, consisting of panels hinged together forming a continuous panel train.  Panels shall be top-supported by one (1) carrier in every other panel, consisting of four (4) permanently lubricated, precision ground ball bearing polished steel wheels riding on a steel tread surface.  Panels shall be operated between stacking location and installed position by an electric operator, which connects to lead panel by #50 roller chain.  A manual override shall be included in the event of a power failure to allow the operable wall system to be manually operated.

- Drive System:  Electric motor shall consist of (select):

  - Standard Electric Motor: consisting of a 1 Horsepower (.746 kw), 115 volt, 1 Phase, 60 Hz capacitance wound motor.

  - Optional Electric Motor: consisting of a 1 Horsepower (.746 kw), 208 volt, 3 Phase, 60 Hz capacitance wound motor.

- Activation of the operator shall be controlled by a two (2) position (low voltage) key switch to arm the system.  Control of the operator shall consist of two (2) stations with extend and retract constant-pressure push button switches.  Switches shall be low voltage, wired in series, and located on opposite sides and ends of the partition.  Electric operator shall include safety devices (limit switches) to automatically shut off the operator at the fully extended and fully retracted position.  Operator shall be located at the opposite end of stack area off center to the side of the partition.  All electric operator components shall be modularized for easy replacement in the field without removing the surrounding components and NFPA 70 approved.  Access panels to the operator unit and return sprockets are required for adjustment and maintenance purposes, as provided by others.  Electric operator shall consist of (select):

  - Standard Speed Reducer Drive: consisting of a 50 to 1 ratio worm gear, adjustable clutch and 1 H.P. (.746 kw) electric motor.  Speed reducer drive system shall be capable of moving a wall system up to 600 ft.2 (56 m2).

2.04 STACK ARRANGEMENTS

A.	Stack Type:  Panel storage configuration shall be Center Stack, consisting of panels stacked on center to the wall's installed position.

B.	Stack Quantity:  Panels shall be stored at (select):

- Standard One End: on one end of the wall run.

2.	Optional Both Ends: on both ends of the wall run.

2.05 FINISHES

A.	Finish Material Type:  Panel finish material shall be Class rated in accordance with ASTM E 84, consisting of (select):

1.	Vinyl: consisting of Type II, reinforced vinyl weighing 21 oz. / lin. yd. (651 g / lin. m).  Vinyl shall meet or exceed CCC-W-408A and CFFA-W-101-D quality standards.

2.	Optional Upgrade Fabric: consisting of fade and tear resistant fabric that resists water-based stains weighing 13 oz. / lin. yd. (403 g / lin. m).

3.	Optional Basics Carpet: consisting of acoustically absorbent, non-woven needle punch fibers fused to prevent fraying and unraveling of material weighing 28.5 oz. / lin. yd. (884 g / lin. m).  Basics Carpet shall achieve a minimum NRC (Noise Reduction Coefficient) rating of .20 (applied over gypsum substrate) in accordance with ASTM C423.

4.	Optional Upgrade Carpet: consisting of acoustically absorbent, non-woven needle punch fibers fused to prevent fraying and unraveling of material weighing 23 oz. / lin. yd. (713 g / lin. m).  Upgrade Carpet shall achieve a minimum NRC (Noise Reduction Coefficient) rating of .25 (applied over gypsum substrate) in accordance with ASTM C423.

5.	Optional Unfinished: consisting of panels with exposed steel skins for field applied wallcovering or painting.

- Finish Material Supplier:  Finish material shall be (select):

- Standard Factory Supplied: from manufacturer’s standard selection of finish materials, as specified.

2.	Optional Customer Supplied: from customer’s selection of finish material, by others, and as approved by KWIK-WALL Company.

C.	Finish Material Application:  Finish material shall be (select):

- Standard Factory Applied: by operable wall manufacturer.  Customer supplied finish material samples must be submitted to manufacturer for testing and approval prior to acceptance and application.

2.	Optional Field Applied: by others.

2.06 PERIMETER TRIM AND SEALS

A.	A.	Vertical Trim and Seals:  Panels shall have vertical astragals containing flexible vinyl seals and incorporate reversible tongue-and-groove-type configurations for positive interlocking with adjacent panels.  Vertical astragal type shall be (select):

- Standard Trimless Astragal: consisting of an aluminum extrusion with tongue-and-groove-type vertical astragals.  Vertical trim shall not be permitted on the panel faces, resulting in a minimal groove appearance between adjacent panels.

2.	Optional Cap-type Astragal: consisting of an aluminum extrusion with tongue-and groove-type vertical astragals for encapsulating and protecting the finish material and substrate along the vertical edge of the panel.

B.	Horizontal Top Trim and Seals:  Top seals shall consist of flexible vinyl sweep seals installed on both sides of the panel.  The seals shall consist of a compressed bulb between two (2) fingers of vinyl.  Top seal type shall be fixed consisting of continuous-contact flexible vinyl, sealing against the bottom flange of the overhead track.

C.	Horizontal Bottom Trim and Seals:  Bottom seals shall consist of multiple fingers of flexible vinyl for positive contact and sealing with various floor surfaces.  Bottom seal type shall be (select):

- Standard Adjustable Bottom Seals: consisting of field-adjustable, continuous-contact vinyl sweep seals with 2” [50.8] nominal height with 3/4" [19] of nominal adjustment.

D.	Horizontal and Vertical Panel Trim:  All exposed panel trim and hinges shall be of one (1) similar color (select):

1.	Dark Bronze.

2.	Grey.

2.07 CLOSURE SYSTEMS

  - Initial Closure System:  The lead panel (the first panel exiting the stack) shall form a seal vertically against a rigid wall surface.  The initial closure shall be accomplished by an Adjustable-Compensating Closure containing two (2) continuous-contact, flexible vinyl bulb seals installed along the vertical edge of the lead panel for positive compression against a rigid wall surface.  Initial closure panel shall contain a flush pull handle on each side.

B.	Final Closure System:  The final closure panel (the last panel exiting the stack) shall form a seal vertically against a rigid wall surface.  Final closure shall be accomplished by a Half Panel that requires attachment to the permanent wall.  The first panel adjacent to the half panel shall contain a flush pull handle.  The type of final closure panel shall be (select):

- Standard Manual Half Panel Pivot Closure: consisting of a Half Panel requiring manual activation when extending or retracting the operable wall.  A half panel inter-lock limit switch shall prevent the operable wall from being retracted until the half panel and adjacent panel are partially folded.

(Note: Minimum of five (5) panels required for Manual Half Panel Pivot Closure system.)

2.08 ACOUSTICAL PERFORMANCE

A.	Certification:  The operable wall shall have been tested in an independent acoustical testing laboratory in accordance with ASTM E 90 and ASTM E 413 test procedures.

B.	STC Rating:  The operable wall acoustical performance rating shall be based on (select):

1.	Standard Steel Skins: with a standard rating of 49 STC, or an optional ratings of 51 STC.

(Note: Not available with optional Wood Veneer or High Pressure Laminate.)

2.	Optional Acoustical Substrate: with optional ratings of 42 STC, 45 STC, 49 STC or 50 STC.

2.09 PANEL ACCESSORIES

A.	Accessories including Single Pass Doors, Keyed Cylinder Locks, Concealed Door Closures, Room Viewers, Exit Signs, Dry Marker Writing Surfaces, Recessed Eraser Trays, Vision Lites, Tack Surfaces, and Pocket Doors shall be compatible with other accessories and options, furnished and installed by the operable wall manufacturer as noted on submitted shop drawings.

2.10 SAFETY DEVICES

A.	Optional safety devices by Guardian Personnel Protection System™. The following safety devices shall shut the operable wall off immediately and prevent any movement until the system is reset at the control box (select):

1.	Floor Pressure Mats: consisting of molded vinyl mats (1/8" [3.18] thick) located in the pocket area and shall shut the operable wall system off immediately if a person or object is placed on mat. Floor mats shall only require 1 lb./ft.2 (4.88 kg/m2) of pressure to activate.

(Note: Mat sizes may require additional width to pocket area.)

2.11 TRACK SYSTEM

A.	Type H.D. Continuously Hinged / Electric Steel Track:  The Continuously Hinged / Electric Steel track running surface shall be made of cold-rolled, high carbon steel tread surfaces to facilitate ease of panel movement and operation.  Track system shall require a panel guide to straighten out the wall system.

  - The steel tread surface shall be contained within a continuous structural track housing extruded from aluminum, which prohibits deterioration caused by rust or corrosion.  The track housing shall have a durable anodized clear satin finish, which resists color fading and flaking.  The track housing shall utilize grooves and interlocking steel pins for positive alignment of adjacent track sections, and shall be reinforced overhead by a heavy-duty steel bracket made of hot-rolled, 3/8" [10] thick plate steel.  Aluminum track housing shall include an integral nut slot to accept a hardened steel square nut to facilitate attachment of each steel all-rod and splice brackets to the overhead structural support.

2.12 CARRIER SYSTEM

A.	Type H.D. Continuously Hinged / Electric Steel Wheel Carrier:  Continuously Hinged / Electric panel shall be top supported by one (1) carrier on every other panel, utilizing a 5/8" [16] diameter pendant bolt.  Each top carrier shall consist of four (4) permanently-lubricated, precision ball bearing polished steel wheels, as required for smooth and quiet operation.

2.13 SUSPENSION SYSTEM

A.	Mounting System:  The track shall be suspended by steel Drop Rods, consisting of adjustable rods of grade 2, 3/8" [10] diameter threaded steel all-rod provided with 3/8" [10] serrated steel nuts.

## PART 3 – EXECUTION

3.01 INSPECTION

A.	Proper and complete preparation of the operable wall system opening shall be by others in accordance with the architectural drawings, manufacturer’s shop drawings and ASTM E 557.  Any deviation of the actual opening from these specifications shall be called to the attention of the architect prior to the installation of the operable wall.

B.	Deficiencies in the operable wall opening shall be corrected by others prior to installation of the operable wall.

3.02 INSTALLATION

A.	The operable wall system shall be installed by manufacturer’s authorized distributor.

B.	The operable wall shall be installed in accordance with manufacturer’s written instructions, shop drawings and ASTM E 557 installation guidelines.

3.03 ADJUSTING AND CLEANING

A.	The operable wall panels and track system shall be adjusted and cleaned in accordance with manufacturer’s written instructions.

3.04 PROTECTION

A.	The operable wall panels shall be stored in the stacked (retracted) position prior to acceptance by the owner's representative.

3.05 DEMONSTRATION

A.	The operable wall manufacturer’s authorized distributor shall demonstrate proper operation and explain proper and necessary maintenance requirements of the operable wall system to the owner's representative.

For additional information contact:

KWIK-WALL Company

4650 Industrial Ave.

Springfield, Illinois 62703

Phone: 217-522-5553 or 800-280-5945 (United States and Canada only)

Fax: 217-522-1170 or 800-290-5945 (United States and Canada only)

Website: www.kwik-wall.com

Email: kwinfo@kwik-wall.com

Note:

Due to ongoing research and development, some variations may occur in product specifications.

3-20