MATA™ – Hinged-Paired Panels

# SECTION 10 22 39 – Folding Glass Partitions

## PART 1 - GENERAL

### DESCRIPTION

    - General

      - Furnish and install operable glass partitions and suspen- sion system. Provide all labor, materials, tools, equip- ment, and services for glass operable walls in accordance with provisions of contract documents.

### RELATED WORK BY OTHERS

    - Preparation of opening will be by General Contractor. Any de- viation of site conditions contrary to approved shop drawings must be called to the attention of the architect.

    - All header, blocking, support structures, jambs, and track en- closures, as required in 1.04 Quality Assurance.

    - Pre-drilling of support structure in accordance with approved shop drawings.

    - Paint or otherwise finishing all trim and other materials adjoin- ing head and jamb of the partitions.

### SUBMITTALS

    - Complete shop drawings are to be provided prior to fabrication indicating construction and installation details.

### QUALITY ASSURANCE

    - Preparation of the opening shall conform to the criteria set forth per ASTM E557 Standard Practice for Architectural Appli- cation and Installation of Operable Partitions.

    - Glass shall be clear tempered or laminated with recent ASTM C1048 and ASTM C1036 standards.

### PRODUCT DELIVERY, STORAGE, AND HANDLING

    - Proper storage of partitions before installation and continued protection during and after installation will be the responsi- bility of the General Contractor. IMPORTANT: Room in which panels are stored and/or installed must be temperature and moisture controlled.

### WARRANTY

Track, carriers, and panel frames shall be guaranteed for one year against defects in material and workmanship. Glass breakage is not included in this warranty unless done during manufacturing or installation processes.

## PART 2 - PRODUCTS

    - Upon compliance with all of the criteria specified in this section, Manufacturers wishing to bid products similar to the product specified must submit to the architect 10 days prior to bidding complete data in support of compliance. The submit- ting manufacturer guarantees the proposed substituted prod- uct complies with the product specified and as detailed on the drawings.

### 2.02 MATERIALS

- Product to be top supported, hinged paired panel, wood- framed movable glass wall MATA™ by Kwik-Wall®

  - Panels shall be nominally 1-3/4” [44mm] thick and 24” [610] to 48” [1219] in width. Maximum standard heights to 10’2” [3099]. Custom heights available but must be approved by the manufacturer.

  - Wood frame of triple laminated solid wood top and bottom rails and stiles that are constructed from custom veneers with an engineered wood core.

    - Basis of design is Red Oak, Maple or Poplar (select one).

    - Optional species:

      - Cherry, Birch, Pine, Walnut and Douglas Fir are available

      - Customer selected species (requires factory approval for manufacturing compatibility)

  - Vertical lead rails shall contain full height partially re- cessed rubber bulb seal and factory provided hinges.

  - Horizontal top & bottom rails shall incorporate continu- ous contact seals of brown multi-ply vinyl.

  - Each panel contains a mortise floor pin which extends into a floor mounted dust covered receptacle to stabilize and secure each panel in the opening.

  - Glass inserts shall be a choice of

    - 1/4” [6] clear tempered glass (standard)

    - 5/16 [8] translucent laminated glass (factory ap- proval required)

    - Custom glass (factory approval required)

  - Glass shall be glazed using matching solid wood stops one side of panel.

  - Panels will be hinged with adjustable mortise hinges or optional concealed hinges on the vertical edges on each pair of panels.

- Weight of the panels shall be approximately: 1/4” [6] tempered glass: 7.0 lbs. per sq. ft. 5/16” [8] laminated glass: 7.5 lbs. per sq. ft.

- Suspension system:

  - Manufacturer supplied -Track shall be of clear anodized architectural grade extruded aluminum alloy 6063-T6. Track design shall provide integral support for adjoining ceiling, soffit, or plenum sound barrier. Track shall be connected to the structural support by pairs of 3/8” [10] dia. threaded steel hanger rods.

    - Each panel shall be supported by one, 4-wheeled carrier. Wheels are made of precision ground steel ball bearings encased with molded polymer tires.

MATA™ – Hinged-Paired Panels

# SECTION 10 22 39 – Folding Glass Partitions

  - Distributor supplied - Customized track and carrier systems supplied by distributor with panels customer fabricated to track and carrier specifications.

- Finishes

  - Aluminum track shall be clear anodized

    - Optional upgrade:

      - Custom anodized

      - Custom powder coated

  - Wood frame shall be furnished stained and finished with a matte polyurethane.

    - Select stain color from manufacturer’s standard stain selection (Red Oak, Maple Frame species only):

- Fruitwood

- Dark Oak

- Wheat

- Cordovan

    - Premium stains available at an upcharge to be selected from the Sherwin Williams® Sher-Wood BAC wiping stains Color Selector.

    - Panels may be ordered unfinished for field finish- ing.

    - Painted solid color: White or Black

    - Custom Color Painted (requires approval of color on veneer sample prior to final finishing and comes at an upcharge.)

[Specifiers Note: All painted systems are fabricated with Poplar frame species as the base.]

  - Hardware finish to be (select one)

    - Polished Brass (standard) (a/k/a Resista)

    - Antique Brass (standard)

    - Satin Nickel (standard)

    - Optional Upgrade (may extend lead times)

      - Polished Chrome

      - Matte Black

      - Oil Rubbed Bronze (a/k/a Rustic Umber)

- Available Accessories/Options

  - Hinged closure panel of the same thickness and con- struction as the basic panels, one-way swing (select as required):

    - Hoppe Munchen style handle

      - Optional upgrade: Lock to be keyed for Schlage brand

  - Floor Locks

    - Adams Rite thumb turn floor lock

    - Adams Rite keyed floor lock

  - Optional glass etching and decals available

### 2.03 OPERATION

- Panels shall be manually moved from the storage area, positioned in the opening, and either edge activated or face activated floorbolts set.

- Final partition closure to be by (select one):

  - Hinged closure with handle

  - Overlapping the opening

  - 2” [51] trail gap

  - Seal to face of storage pocket doors

- Stack/Store Panels

  - Retract floorbolt and move to storage area. Panels are stored at opening perimeter.

## PART 3 - EXECUTION

- Installation. The complete installation of the glass wall system shall be by an authorized installer and be in strict accordance with the approved shop drawings and manufacturer’s standard printed specifications, instructions, and recommendations.

- Cleaning

  - All track and panel surfaces shall be wiped clean and free of handprints, grease, and soil per specific instructions.

  - Optional specialty glass requires special cleaning per instructions provided.

  - Cartoning and other installation debris shall be removed to on-site waste collection area, provided by others.

- Training

  - Installer shall demonstrate proper operation and mainte- nance procedures to owner’s representative.

  - Owners manuals shall be provided to owner’s represen- tative.