ATLAS™ – Individual Multi-Directional Panels, 48, 43 STC

SECTION 10 22 39 FOLDING PANEL PARTITIONS

SECTION 10 22 39.13 FOLDING GLASS-PANEL PARTITIONS

# PART 1 - GENERAL SPECIFICATIONS

## WORK INCLUDED

    - Folding Glass Panel Partition System shall be furnished, installed, and serviced by wall manufacturer’s authorized distributor, in compliance with the architectural drawings and specifications contained herein.

## RELATED WORK

    - Structural Support: Structural support system required for suspending the folding glass panel partition shall be designed, installed and pre-punched by others, in accordance with ASTM E 557 and manufacturer’s shop drawings.

    - Insulation: Sound insulation and baffles for the plenum area above the track system, under the permanent floor, inside air ducts passing over or around the folding glass panel partition, and in permanent walls adjoining the Folding Glass Panel Partition System shall be by others, in accordance with ASTM E 557.

    - Opening Preparation: Proper and complete preparation of the Folding Glass Panel Partition System opening shall be by oth-ers in accordance with ASTM E 557 and shall include floor lev-eling; plumbness of adjoining permanent walls; substrate and/ or ceiling tile enclosures for the track system; and the paint-ing and finishing of trim and other materials adjoining the head and jamb areas of the folding glass panel partition. Any permanent wall(s) receiving an adjustable or fixed wall jamb will require internal structural blocking to secure the jamb to the permanent wall. Refer to a copy of the shop drawings for additional details.

## SYSTEM DESCRIPTION

    - The Folding Glass Panel Partition System shall consist of Indi-vidual Panels that are top supported by two (2) multi-direction-al carriers that are capable of negotiating 90o “X”, “L” and “T” intersections.

    - The Folding Glass Panel Partition System shall consist of acoustically rated panels tested in accordance with ASTM E 90 and ASTM E 413 test procedures and shall have achieved a STC rating as specified herein (see “Acoustical Performance” article listed under Part 2 – Products).

## QUALITY ASSURANCE

    - The folding glass panel partition shall have been tested in an independent acoustical testing laboratory in accordance with ASTM E 90 and ASTM E 413 test procedures.

    - The folding glass panel partition panel construction and finish materials shall consist of Class A-rated materials in accordance with ASTM E 84.

    - The folding glass panel partition shall be installed by the man-ufacturer’s authorized distributor in accordance with ASTM E 557.

## REFERENCES

    - ASTM E 90: Laboratory Measurement of Airborne-Sound Trans-mission Loss of Building Partitions.

    - ASTM E 413: Determination of Sound Transmission Class (STC).

    - ASTM E 557: Architectural Application and Installation of Opera-ble Partitions.

    - ASTM E 84: Surface Burning Characteristics of Building Materi-als.

    - ASTM A 653: Specification for General Requirements for Steel Sheet, Alloy-Coated (Galvannealed) by the Hot Dip Process.

    - ASTM C 423: Standard Test Method for Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method.

    - CCC-W-408A: Federal Specification which applies to Vinyl Coat-ed Wall Coverings.

    - CFFA-W-101-D: Chemical Fabrics and Film Association Quality Standard for Vinyl Coated Fabric Wall Coverings.

    - ATTM E 2190: Certification and testing for Insulated Glass in-serts

## SUBMITTALS

    - Manufacturer shall provide written technical information and related detail drawings, which demonstrate that the products comply with contract documents for each type of operable partition specified.

    - Manufacturer shall provide detailed engineering drawings fea-turing track plan, panel elevation, horizontal and vertical details and beam punching template as required.

    - Manufacturer shall provide written test report of the indepen-dent acoustical testing laboratory certifying the attainment of the specified STC rating, upon request.

    - Manufacturer shall provide written instructions specifying the proper operation and maintenance of the Folding Glass Panel Partition System.

    - Manufacturer shall provide a color selector demonstrating the manufacturer’s selections of the specified finish material.

Samples shall consist of actual swatches of the specified finish material.

## DELIVERY, STORAGE AND HANDLING

    - Panels shall be individually wrapped in a protective plastic covering to keep panels clean during delivery, storage and han-dling.

    - Panels shall be stored on edge and above the floor on cush-ioned blocking in a dry and ventilated area, protected from humidity and temperature extremes.

ATLAS™ – Individual Multi-Directional Panels, 48, 43 STC

SECTION 10 22 39 FOLDING PANEL PARTITIONS

SECTION 10 22 39.13 FOLDING GLASS-PANEL PARTITIONS

## SEQUENCING / SCHEDULING

    - Beam Punching: Manufacturer shall provide beam punch-ing template drawing detailing the anchor locations for the suspended track system (as required for Drop Rod Mounting), as required for the fabrication and installation of structural overhead support by others.

    - Track Installation: Scheduling of folding glass panel partition track installation shall occur after structural overhead support has been properly and completely fabricated and installed by others.

    - Panel Installation: Folding glass panel partition panel installa-tion shall occur after fixed wall substrate construction is prop-erly and completely installed by others, as required to protect panels from ongoing adjacent construction.

## WARRANTY

Manufacturer shall warrant each folding glass panel partition panel and its component parts to be free from defects in material and workmanship for a period of five (5) years from the date of delivery to the original purchaser, when installed by an autho-rized KWIK-WALL distributor. KWIK-WALL also warrants the fixed top seals, track, carriers, and its component parts to be

free from defects in material and workmanship for a period of ten (10) years. (Glass is specifically excluded from the warran-ty.) (Contact your local KWIK-WALL Distributor or KWIK-WALL Company for complete warranty information.)

# PART 2 - PRODUCTS

## ACCEPTABLE MANUFACTURERS

    - Upon compliance with all the criteria specified in this section, Manufacturers wishing to bid products similar to the product specified must submit to the architect 10 days prior to bidding complete data in support of compliance and a list of three past installations of products similar to those listed. The submitting manufacturer guarantees the proposed substituted product complies with the product specified and as detailed on the drawings.

## PANEL CONSTRUCTION

    - Panel Dimensions: Standard panel dimension shall be a nomi-nal 4” [101.6] thick, and nominal 48” wide [1219.2]

    - Panel Frame: Steel frame shall be 16-gauge galvanneal steel, horizontal top cross member shall be minimum 12-gauge galvanneal steel which meets or exceeds ASTM A 653 require-ments. Frame shall be all-welded construction with steel corner supports and cross-bracing reinforcement. Panel frame shall be Class A rated, fire-retardant, non-combustible and

non-corrosive in accordance with ASTM E 84.

    - Panel Skins: Panel skins shall be Class A rated (except Wood Veneer and High-Pressure Laminate) in accordance with ASTM E 84. Panel skin material shall consist of (select):

      - Standard Steel Skins: consisting of minimum 22-gauge tension-leveled galvanneal steel, pressure laminated to a structural acoustical backer and mechanically joined to the steel frame to form a rigid, unitized, and structural panel.

      - Optional Acoustical Substrate: consisting of structural acoustical substrate pressure-laminated to both sides of the steel frame to form a rigid, unitized and structural panel.

      - Optional Wood Veneer: consisting of particle board core covered with wood veneer and pressure laminated to both sides of the steel frame to form a rigid, unitized and structural panel.

      - Optional High-Pressure Laminate: consisting of gypsum board core covered with general-purpose plastic lami-nate and Phenolic backer sheet, which is pressure-lam-inated to both sides of the steel frame to form a rigid, unitized and structural panel.

    - Glass: Opening cut out in panel shall be glazed with insulated glass that is manufactured in accordance with ASTM E 2190. Glass type shall be an acoustical insulated glass unit. Glass shall be retained in the opening cut out using an aluminum extru-sion.

      - 43 STC - 1 3/4” Overall Thickness, 1/2” Laminated Glass - 1” Air Space - 1/4” Tempered Glass

      - 48 STC –1 7/8” Overall Thickness, 1/2” Laminated Glass - 1” Air Space – 3/8” Laminated Glass

    - Panel Weight: Maximum panel weight shall be 10.6 – 10.8 lb./ ft.2 (52 – 53 kg/m2) depending on STC rating, size, and options selected.

## OPERATION

    - Operation shall be Individual Panels with a Multi-Directional track system that allows the panels to negotiate 90o “X”, “L” and “T” intersections as required for movement of panels from storage location(s) to various installed positions. Panels shall be top-supported by two (2) carriers featuring dual horizontal precision bearings with high-strength polymer tires riding on a structural aluminum track.

## STACK ARRANGEMENTS

    - Stack Type: Panel storage configuration shall be (select):

      - Standard Perpendicular Stack: consisting of panels stacked perpendicular to the wall’s installed position.

      - Optional Parallel Stack: consisting of panels stacked par-allel to the wall’s installed position.

ATLAS™ – Individual Multi-Directional Panels, 48, 43 STC

SECTION 10 22 39 FOLDING PANEL PARTITIONS

SECTION 10 22 39.13 FOLDING GLASS-PANEL PARTITIONS

      - Optional Remote Stack: consisting of panels located remotely from the wall’s installed position, as shown on submitted shop drawings.

    - Stack Quantity: Panels shall be stored in separate stack areas as required for panel storage.

## FINISHES

    - Finish Material Type: Panel finish material shall be Class A (except wood veneer and high-pressure laminate) rated in accordance with ASTM E 84, consisting of (select):

      - Vinyl: consisting of Type II, reinforced vinyl weighing 21 oz./lin. yd. (651 g/lin. m). Vinyl shall meet or exceed CCC-W-408A and CFFA-W-101-D quality standards.

      - Optional Upgrade Fabric: consisting of fade and tear-re-sistant fabric that resists water-based stains weighing 13 oz./lin. yd. (403 g/lin. m).

      - Optional Standard Carpet: consisting of acoustically absorbent, non-woven needle punch fibers fused to prevent fraying and unraveling of material weighing 28.5 oz./lin. yd. (884 g/lin. m). Basics Carpet shall achieve a minimum NRC (Noise Reduction Coefficient) rating of

.20 (applied over gypsum substrate) in accordance with ASTM C423.

      - Optional Upgrade Carpet: consisting of acoustically absorbent, non-woven needle punch fibers fused to prevent fraying and unraveling of material weighing 23 oz./lin. yd. (713 g/lin. m). Upgrade Carpet shall achieve a minimum NRC (Noise Reduction Coefficient) rating of

.25 (applied over gypsum substrate) in accordance with ASTM C423.

      - Optional Wood Veneer: consisting of unfinished flat cut wood veneer laminated to 1/2” [12.7] thick particle board core. Veneer shall be book / running matched within a panel, and edge-banded if trimless astragals are speci-fied.

      - Optional High-Pressure Laminate: consisting of gyp-sum board core covered with general-purpose plastic laminate and Phenolic backer sheet, which is pressure laminated to both sides of the steel frame to form a rigid, unitized, and structural panel.

      - Optional Unfinished: consisting of panels with exposed acoustical substrate or steel skins for field-applied wall covering or painting.

    - Finish Material Supplier: Finish material shall be (select):

      - Standard Factory Supplied: from manufacturer’s stan-dard selection of finish materials, as specified.

      - Optional Customer Supplied: from customer’s selection of finish material, by others, and as approved by KWIK-WALL Company.

    - Finish Material Application: Finish material shall be (select):

      - Standard Factory Applied: by folding glass panel partition manufacturer. Customer-supplied finish material sam-ples must be submitted to manufacturer for testing and approval prior to acceptance and application.

      - Optional Field Applied: by others.

## PERIMETER TRIM AND SEALS

    - Vertical Trim and Seals: Panels shall have vertical astragals con-taining flexible vinyl seals and incorporate reversible tongue-and-groove-type configurations for positive interlocking with adjacent panels. Vertical astragal type shall be (select):

      - Standard Trimless Astragal: consisting of an aluminum extrusion with tongue-and-groove-type vertical astragals. Vertical trim shall not be permitted on the panel faces, resulting in a minimal groove appearance between adja-cent panels.

      - Optional Cap-type Astragal: consisting of an aluminum extrusion with tongue-and groove-type vertical astragals for encapsulating and protecting the finish material and substrate along the vertical edge of the panel.

    - Horizontal Top Trim and Seals: Top seals shall consist of flexible vinyl sweep seals installed on both sides of the panel. The seals shall consist of a compressed bulb between two (2) fingers of vinyl. Top seal type shall be (select):

      - Standard Fixed Top Seals: consisting of continuous-con-tact flexible vinyl sealing against the bottom flange of the overhead track.

    - Horizontal Bottom Trim and Seals: Bottom seals shall consist of multiple fingers of flexible vinyl for positive contact and sealing with various floor surfaces. Bottom seal type shall be (select):

      - Standard Operable Bottom Seals: consisting of an edge-activated seal using a removable wrench as sup-plied by manufacturer. Bottom seals shall provide 2” [50.8] of nominal travel.

      - Optional Adjustable Bottom Seals: consisting of field-ad-justable, continuous-contact vinyl sweep with 2” [50.8] nominal height with ¾” [19] of adjustment.

      - Optional Automatic Bottom Seals: consisting of self-acti-vated seals providing 2” [50.8] of nominal travel.

    - Horizontal and Vertical Panel Trim: All exposed panel trim and hinges shall be of one (1) similar color (select):

      - Dark Bronze.

      - Grey.

## CLOSURE SYSTEMS

ATLAS™ – Individual Multi-Directional Panels, 48, 43 STC

SECTION 10 22 39 FOLDING PANEL PARTITIONS

SECTION 10 22 39.13 FOLDING GLASS-PANEL PARTITIONS

    - Initial Closure System: The lead panel (the first panel exiting the stack) shall form a seal vertically against a rigid wall surface, as accomplished by a (select):

      - Standard Bulb Seal: consisting of continuous-contact, flexible vinyl bulb seals installed along the vertical edge of the lead panel for positive compression against a rigid wall surface.

      - Optional Fixed Starter Jamb: consisting of an aluminum extrusion, which is permanently mounted to a structural wall surface. The Fixed Starter Jamb shall incorporate

a tongue-and-groove-type vertical astragal for positive interlocking with the lead panel.

      - Optional Adjustable Starter Jamb: consisting of an alu-minum extrusion which is permanently mounted to a structural wall surface and is field-adjustable to compen-sate for out-of-plumb conditions of the fixed wall. The Adjustable Starter Jamb shall incorporate a tongue-and-groove-type vertical astragal for positive interlocking with the lead panel.

    - Final Closure System: The final closure panel (the last panel exiting the stack) shall form a seal vertically against a rigid wall surface. The type of final closure panel shall be (select):

      - Standard Expander Panel Closure: consisting of an expander mechanism with a nominal 5” [127] of travel, activated from the face of the panel using a removable wrench as supplied by manufacturer. The Expander Panel shall be equipped with an adjustable bottom seal (standard) or (optional) operable bottom seal, and a flush pull handle.

      - Optional Hinged Panel(s) Closure: consisting of a panel hinged permanently and directly to a structural wall surface. The Hinged Panel(s) shall be equipped with an adjustable bottom seal, a lap-type extrusion for sealing against its adjacent panel (standard) or (optional) ex-pander mechanism with a nominal 5” [127] of travel, and a flush pull handle on each side of the panel.

      - Optional Pocket Door(s): (see “Series 3000 Pocket Door” technical data sheet for complete details and specifica-tions).

## ACOUSTICAL PERFORMANCE

    - Certification: The folding glass panel partition shall have been tested in an independent acoustical testing laboratory in accor-dance with ASTM E 90 and ASTM E 413 test procedures.

    - STC Rating: The folding glass panel partition acoustical perfor-mance rating shall be based on (select):

      - Standard Steel Skins: with a standard rating of 48 and 43 STC.

## PANEL ACCESSORIES

    - Accessories including Pass Doors; Single or Double, Keyed Cyl-inder Locks, Concealed Door Closures, and Pocket Doors shall be compatible with other accessories and options, furnished and installed by the folding glass panel partition manufacturer as noted on submitted shop drawings.

## TRACK SYSTEMS

- Track Type: The folding glass panel partition track system shall be extruded from structural aluminum alloy, which prohib-

its deterioration caused by rust or corrosion. The aluminum track shall have a durable anodized clear satin finish, which resists color fading and flaking. The track shall utilize grooves and interlocking steel pins for positive alignment of adjacent track sections. The track joints shall be reinforced overhead by a heavy-duty steel bracket made of hot-rolled, 3/8” [10] thick plate steel. Aluminum track shall include an integral nut slot to accept a hardened steel square nut to facilitate attachment of each steel all-rod and splice brackets to the overhead structural support.

- Track Size: The track size shall be (selected from Track and Car-rier Selection Chart - refer to Page 1):

  - Type 425 Multi-Directional Aluminum Track: certified to be capable of supporting up to 525 lb. (238 kg) of total live load weight per panel.

  - Type 850 Multi-Directional Aluminum Track: certified to be capable of supporting up to 850 lb. (386 kg) of total live load weight per panel.

## CARRIER SYSTEMS

    - Carrier Type: Each Individual Panel shall be top-supported by two (2) carriers utilizing a 5/8” [16] diameter pendant bolt. Each carrier shall consist of dual horizontal, permanently lubricated, precision-ground steel bearings with high-strength polymer tires, as required for smooth and quiet operation. Multi-Direc-tional carriers shall be capable of negotiating 90o “X”, “L” and “T” intersections as required for moving panels from storage location(s) to various installed positions.

    - Carrier Size: The carrier size shall be (select from Track and Carrier Selection Chart - refer to Page 1):

      - Type 425 Multi-Directional Carrier: certified to be capable of supporting up to 525 lb. (238 kg) of total live load per panel.

      - Type 850 Multi-Directional Carrier: certified to be capable of supporting up to 850 lb. (386 kg) of total live load per panel.

ATLAS™ – Individual Multi-Directional Panels, 48, 43 STC

SECTION 10 22 39 FOLDING PANEL PARTITIONS

SECTION 10 22 39.13 FOLDING GLASS-PANEL PARTITIONS

## SUSPENSION SYSTEMS

    - Mounting Systems: The track shall be supported by (select):

      - Standard Drop Rod Mount: consisting of adjustable rods of grade 2, 3/8” [10] diameter threaded steel all-rod pro-vided with 3/8” [10] serrated steel nuts.

      - Optional Direct Mount: consisting of 3/8” [10] x 3” [76] lag screws for attachment to an overhead structural (wood) support. (Direct mount track installations should not exceed 425 lb. (193 kg) of panel weight.)

      - Optional Drop Rod Bracket Mount: consisting of 3/8” [10] thick steel brackets mounted to top of track and sup-ported with adjustable rods of grade 2, 3/8” [10] diameter threaded steel all-rod provided with 3/8” [10] serrated steel nuts

# PART 3 - EXECUTION

## INSPECTION

          - Proper and complete preparation of the Folding Glass Panel Partition System opening shall be by others in accordance with the architectural drawings, the manufacturer’s shop drawings and ASTM E 557. Any deviation of the actual opening from these specifications shall be called to the attention of the archi-tect prior to the installation of the folding glass panel partition.

          - Deficiencies in the folding glass panel partition opening shall be corrected by others prior to installation of the folding glass panel partition.

## INSTALLATION

          - The Folding Glass Panel Partition System shall be installed by the manufacturer’s authorized distributor.

          - The folding glass panel partition shall be installed in accor-dance with the manufacturer’s written instructions, shop draw-ings and ASTM E 557 installation guidelines.

## ADJUSTING AND CLEANING

          - The folding glass panel partition panels and track system shall be adjusted and cleaned in accordance with manufacturer’s written instructions.

## PROTECTION

          - The folding glass panel partition panels shall be stored in the stacked (retracted) position prior to acceptance by the owner’s representative.

## DEMONSTRATION

          - The folding glass panel partition manufacturer’s authorized distributor shall demonstrate proper operation and explain proper and necessary maintenance requirements of the Fold-ing Glass Panel Partition System to the owner’s representative.

For additional information contact:

KWIK-WALL Company 4650 Industrial Ave.

Springfield, Illinois 62703

Phone: 217-522-5553 or 800-280-5945 (United States and Canada only)

Fax: 217-522-1170 or 800-290-5945 (United States and Canada only)

Website: www.kwik-wall.com Email: info@kwik-wall.com

Note:

Due to ongoing research and development, some variations may occur in product specifications.

04-2026