MODEL 2020 — INDIVIDUAL, MULTI-DIRECTIONAL PANELS

SECTION 10 22 26 OPERABLE PARTITIONS

SECTION 10 22 39 FOLDING PANEL PARTITIONS

# PART 1 – GENERAL SPECIFICATIONS

## WORK INCLUDED

    - Operable wall system shall be furnished, installed, and serviced by wall manufacturer’s authorized distributor, in compliance with the architectural drawings and specifications contained herein.

## RELATED WORK

    - Structural Support: Structural support system required for suspending the operable wall shall be designed, installed, and pre-punched by others, in accordance with ASTM E 557 and manufacturer’s shop drawings.

    - Insulation: Sound insulation and baffles for the plenum area above the track system, under the permanent floor, inside air ducts passing over or around the operable wall, and in per-manent walls adjoining the operable wall system shall be by others, in accordance with ASTM E 557.

    - Opening Preparation: Proper and complete preparation of the operable wall system opening shall be by others in accordance with ASTM E 557, and shall include floor leveling; plumbness

of adjoining permanent walls; substrate and/or ceiling tile en-closures for the track system; and the painting and finishing of trim and other materials adjoining the head and jamb areas of the operable wall. Any permanent wall(s) receiving an adjust-able or fixed wall jamb will require internal structural blocking to secure the jamb to the permanent wall. Refer to manufac-turer’s shop drawings for additional details.

## SYSTEM DESCRIPTION

    - The operable wall system shall consist of Individual Panels that are top supported by two (2) multi-directional carriers that are capable of negotiating 90o “X”, “L” and “T” intersections.

    - The operable wall system shall consist of acoustically rated panels tested in accordance with ASTM E 90 and ASTM E 413 test procedures, and shall have achieved a STC rating as spec-ified herein (see “Acoustical Performance” article listed under Part 2 – Products).

## QUALITY ASSURANCE

    - The operable wall shall have been tested in an independent acoustical testing laboratory in accordance with ASTM E 90 and ASTM E 413 test procedures.

    - The operable wall panel construction and finish materials shall consist of Class A-rated materials (except as noted, under “Fin-ishes” Part 2 – Products) in accordance with ASTM E 84.

    - The operable wall shall be installed by the manufacturer’s au-thorized distributor in accordance with ASTM E 557.

## REFERENCES

    - ASTM E 90: Laboratory Measurement of Airborne-Sound Trans-mission Loss of Building Partitions.

    - ASTM E 413: Determination of Sound Transmission Class (STC).

    - ASTM E 557: Architectural Application and Installation of Opera-ble Partitions.

    - ASTM E 84: Surface Burning Characteristics of Building Materi-als.

    - ASTM A 653: Specification for General Requirements for Steel Sheet, Alloy-Coated (Galvannealed) by the Hot Dip Process.

    - ASTM C 423: Standard Test Method for Sound Absorption and Sound Absorption Coefficients by the Reverberation Room Method.

    - CCC-W-408A: Federal Specification which applies to Vinyl Coat-ed Wall Coverings.

    - CFFA-W-101-D: Chemical Fabrics and Film Association Quality Standard for Vinyl Coated Fabric Wall Coverings.

## SUBMITTALS

    - Manufacturer shall provide written technical information and related detail drawings, which demonstrate that the products comply with contract documents for each type of operable partition specified.

    - Manufacturer shall provide detailed engineering drawings featuring track plan, panel elevation, horizontal and vertical details, and beam punching template as required.

    - Manufacturer shall provide a written test report of the indepen-dent acoustical testing laboratory certifying the attainment of the specified STC rating, upon request.

    - Manufacturer shall provide written instructions specifying the proper operation and maintenance of the operable wall sys-tem.

    - Manufacturer shall provide a color selector demonstrating the manufacturer’s selections of the specified finish material.

Samples shall consist of actual swatches of the specified finish material.

## SUBMITTALS

- KWIK-WALL shall provide written technical information and related detail drawings, which demonstrate that the products comply with contract documents for each type of operable partition specified.

- KWIK-WALL shall provide detailed engineering drawings fea-turing track plan, panel elevation, horizontal and vertical details and beam punching template as required.

- KWIK-WALL shall provide written test report of the indepen-dent acoustical testing laboratory certifying the attainment of the specified STC rating, upon request.

MODEL 2020 — INDIVIDUAL, MULTI-DIRECTIONAL PANELS

SECTION 10 22 26 OPERABLE PARTITIONS

SECTION 10 22 39 FOLDING PANEL PARTITIONS

- KWIK-WALL shall provide written instructions specifying the proper operation and maintenance of the operable wall sys-tem.

- KWIK-WALL shall provide a color selector demonstrating the manufacturer’s selections of the specified finish material. Samples shall consist of actual swatches of the specified finish material.

## DELIVERY, STORAGE, AND HANDLING

    - Panels shall be individually wrapped in a protective plastic covering to keep panels clean during delivery, storage, and handling.

    - Panels shall be stored on edge and above the floor on cush-ioned blocking in a dry and ventilated area, protected from humidity and temperature extremes.

## SEQUENCING / SCHEDULING

    - Beam Punching: Manufacturer shall provide beam punch-ing template drawing detailing the anchor locations for the suspended track system (as required for Drop Rod Mounting), as required for the fabrication and installation of structural overhead support by others.

    - Track Installation: Scheduling of operable wall track installation shall occur after structural overhead support has been properly and completely fabricated and installed by others.

    - Panel Installation: Operable wall panel installation shall occur after fixed wall substrate construction is properly and com-pletely installed by others, as required to protect panels from ongoing adjacent construction.

## WARRANTY

Manufacturer shall warrant each operable wall panel and its component parts to be free from defects in material and work-manship for a period of ten (10) years from the date of delivery to the original purchaser, when installed by an authorized KWIK-WALL distributor.

KWIK-WALL also warrants the fixed top seals, track, carriers, and its component parts to be free from defects in material and workmanship for a period of ten (10) years. (Contact your local KWIK-WALL Distributor or KWIK-WALL Company for com-plete warranty information.) (Glass is specifically excluded from the warranty.)

# PART 2 – PRODUCT SPECIFICATIONS

## ACCEPTABLE MANUFACTURER

    - Operable walls shall be Series 2000, Model 2020 Individual Panels / Multi-Directional as manufactured by KWIK-WALL Company.

## PANEL CONSTRUCTION

    - Panel Dimensions: Standard panel dimension shall be a nomi-nal 3” [76] thick.

    - Panel Frame: Vertical steel frame members shall be mini-mum 18-gauge galvanneal steel, horizontal top cross member shall be minimum 12-gauge galvanneal steel, which meets or exceeds ASTM A 653 requirements. Frame shall be all-welded construction with steel corner supports and cross-bracing rein-forcements. Panel frame shall be Class A-rated fire-retardant, non-combustible, and non-corrosive in accordance with ASTM E 84.

    - Panel Skins: Panel skins shall be Class A-rated (except Wood Veneer and High-Pressure Laminate) in accordance with ASTM E 84. Panel skin material shall consist of (select):

      - Acoustical Substrate: consisting of structural acoustical substrate pressure laminated to both sides of the steel frame to form a rigid, unitized, and structural panel.

      - Steel Skins: consisting of minimum 22-gauge tension-lev-eled galvanneal steel, pressure laminated to a structural acoustical backer and mechanically joined to the steel

frame to form a rigid, unitized, and structural panel.

      - Wood Veneer: consisting of particle board core covered with wood veneer and pressure laminated to both sides of the steel frame to form a rigid, unitized and structural panel.

      - High-Pressure Laminate: consisting of gypsum board core covered with general-purpose plastic laminate and Phenolic backer sheet, which is pressure laminated to both sides of the steel frame to form a rigid, unitized, and structural panel.

*Wood Veneer or High-Pressure Laminate is only available with Acoustical Substrate Construction.

    - Insulated Glass vision inserts: Opening cut out in panel shall be glazed with insulated glass that is manufactured in accordance with ASTM E 2190. Glass type shall be an acoustical insulated glass unit. Glass shall be retained in the opening cut out using an aluminum extrusion.

    - Panel Hinges (if required): Panel hinges shall be architectural grade, full-leaf butt hinges. Hinges shall be attached to the steel frame of the panel, and reinforced with a steel backer plate.

    - Panel Weight: Maximum panel weight shall be 6.5 – 9.2 lb./ ft.2 (32 – 45 kg/m2) depending on STC rating, size and options selected.

MODEL 2020 — INDIVIDUAL, MULTI-DIRECTIONAL PANELS

SECTION 10 22 26 OPERABLE PARTITIONS

SECTION 10 22 39 FOLDING PANEL PARTITIONS

## OPERATION

    - Operation shall be Individual Panels with a Multi-Directional track system that allows the panels to negotiate 90o “X”, “L” and “T” intersections as required for movement of panels from storage location(s) to various installed positions. Panels shall be top-supported by two (2) carriers featuring dual horizontal precision bearings with high-strength polymer tires riding on a structural aluminum track.

## STACK ARRANGEMENTS

    - Stack Type: Panel storage configuration shall be (select):

      - Perpendicular Stack: consisting of panels stacked per-pendicular to the wall’s installed position.

      - Parallel Stack: consisting of panels stacked parallel to the wall’s installed position.

      - Remote Stack: consisting of panels located remotely

from the wall’s installed position, as shown on submitted shop drawings.

    - Stack Quantity: Panels shall be stored at one (1) or both ends in separate stack areas as required for panel storage.

## FINISHES

    - Finish Material Type: Panel finish material shall be Class A (except wood veneer and high-pressure laminate) rated in accordance with ASTM E 84, consisting of (select):

      - Vinyl: consisting of Type II, reinforced vinyl weighing 21 oz./lin. yd. (651 g/lin. m). Vinyl shall meet or exceed CCC-W-408A and CFFA-W-101-D quality standards.

      - Fabric: consisting of fade and tear-resistant fabric that resists water-based stains weighing 13 oz./lin. yd. (403 g/ lin. m).

      - Carpet: consisting of acoustically absorbent, non-wo-ven needle punch fibers fused to prevent fraying and

unraveling of material weighing 28.5 oz./lin. yd. (884 g/lin. m). Basics Carpet shall achieve a minimum NRC (Noise Reduction Coefficient) rating of .20 (applied over gypsum substrate) in accordance with ASTM C 423.

      - Upgrade Carpet: consisting of acoustically absorbent, non-woven needle punch fibers fused to prevent fraying and unraveling of material weighing 23 oz./lin. yd. (713 g/ lin. m). Upgrade Carpet shall achieve a minimum NRC (Noise Reduction Coefficient) rating of .25 (applied over gypsum substrate) in accordance with ASTM C 423.

      - Wood Veneer: consisting of unfinished flat cut wood veneer laminated to 1/2” [12.7] thick particle board core. Veneer shall be book and running matched within a panel, and vertically edge-banded if trimless astragals are specified.

(Notes: Optional Class “A” rated particle board is available.

Acoustical substrate STC ratings apply for Wood Veneer panel construction.)

      - High-Pressure Laminate: consisting of gypsum board core covered with general-purpose plastic laminate and Phenolic backer sheet, which is pressure laminated to both sides of the steel frame to form a rigid, unitized, and structural panel.

(Note: Acoustical substrate STC ratings apply for High Pressure Laminate panel construction.)

      - Digitally Printed Steel Skins: Seven-ply construction com-prised of melted, rolled, coated, or printed layers. (52 and 50 STC steel skin panel construction only)

      - Unfinished: consisting of panels with exposed acoustical substrate or steel skins for field-applied wall covering or painting.

    - Finish Material Supplier: Finish material shall be (select):

      - Factory Supplied: from manufacturer’s standard selec-tion of finish materials, as specified.

      - Optional Customer Supplied: from customer’s selection of finish material, by others, and as approved by KWIK-WALL Company.

    - Finish Material Application: Finish material shall be (select):

      - Factory Applied: by operable wall manufacturer. Cus-tomer-supplied finish material samples must be sub-mitted to manufacturer for testing and approval prior to acceptance and application.

      - Optional Field Applied: by others.

## PERIMETER TRIM AND SEALS

    - Vertical Trim and Seals: Panels shall have vertical astragals con-taining flexible vinyl seals and incorporate reversible tongue-and-groove-type configurations for positive interlocking with adjacent panels. Vertical astragal type shall be (select):

      - Trimless Astragal: consisting of an aluminum extrusion with tongue-and-groove-type vertical astragals. Vertical trim shall not be permitted on the panel faces, resulting in a minimal groove appearance between adjacent pan-els.

      - Cap-type Astragal: consisting of an aluminum extru-sion with tongue-and-groove-type vertical astragals for encapsulating and protecting the finish material and substrate along the vertical edge of the panel.

    - Horizontal Top Trim and Seals: Top seals shall consist of flexible vinyl sweep seals installed on both sides of the panel. The seals shall consist of a compressed bulb between two (2) fingers of vinyl. Top seal type shall be (select):

      - Fixed Top Seals: consisting of continuous-contact flexible vinyl sealing against the bottom flange of the overhead track.

MODEL 2020 — INDIVIDUAL, MULTI-DIRECTIONAL PANELS

SECTION 10 22 26 OPERABLE PARTITIONS

SECTION 10 22 39 FOLDING PANEL PARTITIONS

      - Operable Top Seals: consisting of an edge-activated seal using a removable wrench as supplied by manufacturer. Top seals shall provide a maximum ½” [13] of travel.

    - Horizontal Bottom Trim and Seals: Bottom seals shall consist of multiple fingers of flexible vinyl for positive contact and sealing with various floor surfaces. Bottom seal type shall be (select):

      - Operable Bottom Seals: consisting of an edge-activated seal using a removable wrench as supplied by manu-facturer. Bottom seals shall provide 2” [50.8] of nominal travel.

      - Adjustable Bottom Seals: consisting of field-adjustable, continuous-contact vinyl sweep seals with 2” [50.8] nomi-nal height with 3/4” [19] of nominal adjustment.

      - Automatic Bottom Seals: consisting of self-activated seals providing 2” [50.8] of nominal travel.

    - Horizontal and Vertical Panel Trim: All exposed panel trim and hinges shall be of one (1) similar color, as selected from manu-facturer’s standard colors.

## CLOSURE SYSTEMS

    - Initial Closure System: The lead panel (the first panel exiting the stack) shall form a seal vertically against a rigid wall surface, as accomplished by (select):

      - Bulb Seal: consisting of continuous-contact, flexible vinyl bulb seals installed along the vertical edge of the lead panel for positive compression against a rigid wall sur-face.

      - Fixed Starter Jamb: consisting of an aluminum extrusion, which is permanently mounted to a structural wall sur-face. The Fixed Starter Jamb shall incorporate a tongue-and-groove-type vertical astragal for positive interlocking with the lead panel.

      - Adjustable Starter Jamb: consisting of an aluminum extrusion which is permanently mounted to a structural wall surface and is field-adjustable to compensate for

out-of-plumb conditions of the fixed wall. The Adjustable Starter Jamb shall incorporate a tongue-and-groove-type vertical astragal for positive interlocking with the lead panel.

    - Final Closure System: The final closure panel (the last panel exiting the stack) shall form a seal vertically against a rigid wall surface. The type of final closure panel shall be (select):

      - Expander Panel Closure: consisting of an expander mechanism with a nominal 5” [127] of travel, activated

from the face of the panel using a removable wrench as supplied by manufacturer. The Expander Panel shall be equipped with an adjustable bottom seal (standard) or (optional) operable bottom seal, and a flush pull handle.

      - Hinged Panel(s) Closure: consisting of a panel hinged permanently and directly to a structural wall surface. The Hinged Panel(s) shall be equipped with an adjustable bottom seal, a lap-type extrusion for sealing against its adjacent panel (standard) or (optional) expander mecha-nism with a nominal 5” [127] of travel, activated from the face of the panel using a removable wrench, and a flush pull handle on each side of the panel.

      - Pocket Door(s): (see “Series 2000 Pocket Door” brochure for complete details and specifications).

Note: Automatic Bottom Seal is not available in conjunc-tion with Final Closure Panel(s).

## ACOUSTICAL PERFORMANCE

    - Certification: The operable wall shall have been tested in an independent acoustical testing laboratory in accordance with ASTM E 90 and ASTM E 413 test procedures.

    - STC Rating: The operable wall acoustical performance rating shall be based on (select):

      - Steel Skins: with available, certified STC ratings of 50 and 52.

      - Acoustical Substrate: with available, certified STC ratings of 42,45,49 and 50.

(Note: Not available with optional Wood Veneer or High-Pressure Laminate.)

## PANEL ACCESSORIES

    - Accessories, including Pass Doors; Single or Double, Keyed Cylinder Locks, Concealed Door Closures, Room Viewers, Exit Signs, Dry Marker Writing Surfaces, Recessed Eraser Trays, Vi-sion Lites, Tack Surfaces, and Pocket Doors, shall be compatible with other accessories and options, furnished and installed by the operable wall manufacturer as noted on submitted shop drawings.

## TRACK SYSTEMS

- Track Type: The operable wall track system shall be extruded from structural aluminum alloy, which prohibits deteriora-tion caused by rust or corrosion. Aluminum track shall have a durable, clear, satin anodized finish that resists color fading and flaking. The track shall utilize grooves and interlocking steel pins for positive alignment of adjacent track sections. The track joints shall be reinforced overhead by a heavy-du-ty steel bracket made of hot-rolled, 3/8” [10] thick plate steel.

Aluminum track shall include an integral nut slot to accept a hardened steel square nut for attachment of each steel all-rod and splice brackets to the overhead structural support.

MODEL 2020 — INDIVIDUAL, MULTI-DIRECTIONAL PANELS

SECTION 10 22 26 OPERABLE PARTITIONS

SECTION 10 22 39 FOLDING PANEL PARTITIONS

- Track Size: The track size shall be (selected from Track and Car-rier Selection Chart - refer to Page 1):

  - Type 425 Multi-Directional Aluminum Track: certified to be capable of supporting up to 525 lb. (238 kg) of total live load weight per panel.

  - Type 850 Multi-Directional Aluminum Track: certified to be capable of supporting up to 850 lb. (386 kg) of total live load weight per panel.

## CARRIER SYSTEMS

    - Carrier Type: Each individual panel shall be top supported by two (2) carriers utilizing a 5/8” [16] diameter pendant bolt. Each carrier shall consist of dual horizontal, permanently lubricated, precision-ground steel bearings with high-strength polymer tires as required for smooth and quiet operation. Multi-Direc-tional carriers shall be capable of negotiating 90o “X”, “L” and “T” intersections as required for moving panels from storage location(s) to various installed positions.

    - Carrier Size: The carrier size shall be (select from Track and Carrier Selection Chart - refer to Page 1):

      - Type 425 Multi-Directional Carrier: certified to be capable of supporting up to 525 lb. (238 kg) of total live load weight per panel.

      - Type 850 Multi-Directional Carrier: certified to be capable of supporting up to 850 lb. (386 kg) of total live load weight per panel.

## SUSPENSION SYSTEMS

    - Mounting Systems: The track shall be supported by (select):

      - Drop Rod Mount to track square nut raceway: consisting of adjustable rods of grade 2, 3/8” [10] diameter threaded steel all-rod provided with 3/8” [10] serrated steel nuts. Note: Standard rod spacing width is 2.5” on each side of the opening run centerline.

      - Drop Rod Bracket Mount : consisting of 3/8” [10] thick steel brackets by 8” wide, mounted to the square nut integral raceways at the top of track, and suspended with adjustable rods of grade 2, 3/8” [10] diameter threaded steel all-thread rod provided with 3/8” [10] serrated steel flange nuts.

      - Direct Mount: consisting of 3/8” [10] x 3” [76] lag screws for attachment to an overhead structural (wood) support. (Direct mount track installations should not exceed 425 lb. (193 kg) of panel weight.)

# PART 3 – EXECUTION

## INSPECTION

          - Proper and complete preparation of the operable wall system opening shall be by others in accordance with the architectural drawings, manufacturer’s shop drawings, and ASTM E 557. Any deviation of the actual opening from these specifications shall be called to the attention of the architect prior to the installa-tion of the operable wall.

          - Deficiencies in the operable wall opening shall be corrected by others prior to installation of the operable wall.

## INSTALLATION

          - The operable wall system shall be installed by manufacturer’s authorized distributor.

          - The operable wall shall be installed in accordance with manu-facturer’s written instructions, shop drawings and ASTM E 557 installation guidelines.

## ADJUSTING AND CLEANING

          - The operable wall panels and track system shall be adjusted and cleaned in accordance with manufacturer’s written in-structions.

## PROTECTION

          - The operable wall panels shall be stored in the stacked (retract-ed) position prior to acceptance by the owner’s representative.

## DEMONSTRATION

          - The operable wall manufacturer’s authorized distributor shall demonstrate proper operation and explain proper and neces-sary maintenance requirements of the operable wall system to the owner’s representative.

For additional information contact:

KWIK-WALL Company 900 S. Cain St.

Clinton, IL 61727

Phone: 217-522-5553 or 800-280-5945 (United States and Canada only)

Fax: 217-522-1170 or 800-290-5945 (United States and Canada only) Website: www.kwik-wall.com

Email: info@kwik-wall.com

Note:

Due to ongoing research and development, product specifications may vary.

5.2026